DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CSA07 – COMPUTER NETWORKS

COMPUTER NETWORKS ASSIGNMENT

Topology, Addressing, Congestion Control and Application-Service Design for a Bank's Core-Banking, Branch and ATM Network


Submitted by:
MONISH L (1972260035)

Course: CSA07 – Computer Networks

Academic Year: 2026

# Problem Statement

Meridian Bank is upgrading the core-banking network that connects four branch offices, a citywide ATM

network, and a central data center. The network must carry real-time transaction traffic between

branches/ATMs and the data center, ATM surveillance camera feeds, and online-banking portal traffic, while

meeting strict reliability and latency requirements for financial transactions and continuing to operate if a

branch&#39;s primary WAN link goes down. Before the upgrade is deployed, your team, acting as the bank&#39;s network

design group, must design, simulate, and justify the physical topology, addressing and routing scheme,

transport-layer behavior, and application-layer services for this network.

# Abstract

This report presents a proposed computer-network design for Meridian Bank, a banking organization that connects four branch offices, a citywide ATM network, a central data center, surveillance systems, and an online-banking portal. The design focuses on reliable communication, fault tolerance, correct IPv4 addressing, dynamic routing, transport-layer selection, congestion control, quality of service, application services, and network-performance measurement.

A hybrid topology is proposed. Star topology is used within branches, while redundant point-to-point WAN connectivity is used between branches and the central data center. The addressing plan uses private IPv4 space and subnetting. OSPF is selected as the dynamic routing protocol because it can calculate alternate paths when a primary link becomes unavailable. CRC is used as the data-link error-detection example.

TCP is selected for financial transactions and online banking because reliable and ordered delivery is essential. UDP is selected for surveillance camera traffic because continuous delivery and low delay are more important than retransmitting every lost video packet. QoS and traffic shaping are used to protect transaction traffic during congestion. The report also describes an end-to-end online-banking exchange involving DNS, TCP, TLS/HTTPS, the application server, and database server.

The assignment requires simulation of normal operation and a degraded-link condition, with throughput, end-to-end delay, jitter, and packet loss measured in both cases. The numerical performance figures included in this report are clearly identified as illustrative expected values; actual measurements should be replaced with results captured from the team's simulator.

# Table of Contents

- 1. Introduction and Problem Statement

- 2. Objectives

- 3. Requirements and Environment

- 4. Proposed Network Architecture

- 5. Physical Topology and Devices

- 6. Transmission Media and Cabling

- 7. OSI Layer Mapping

- 8. Data-Link Error Detection Using CRC

- 9. IPv4 Addressing and Subnetting

- 10. Routing Design Using OSPF

- 11. Transport-Layer Design

- 12. Congestion Control and QoS

- 13. Application-Layer Services

- 14. End-to-End Online Banking Message Trace

- 15. Security Design

- 16. Simulation Plan

- 17. Test Cases

- 18. Performance Metrics and Analysis

- 19. Failure and Recovery Analysis

- 20. Advantages and Limitations

- 21. SDG Relevance

- 22. Learning Outcomes and Reflection

- 23. Individual Contribution

- 24. Conclusion

- 25. References

- Appendix A: Configuration Templates

- Appendix B: One-Page Executive Summary

# 1. Introduction and Problem Statement

Meridian Bank is upgrading the network infrastructure that connects four branch offices, a citywide ATM network and a central data center. The network must carry real-time transaction traffic, surveillance camera feeds and online-banking portal traffic. The environment therefore requires a design that balances availability, reliability, security, performance and scalability.

A major design requirement is continued operation when a branch's primary WAN link fails. A network that depends on a single WAN path would create a service interruption whenever that path becomes unavailable. For a bank, such an interruption can prevent transactions, affect customers and disrupt branch operations. The proposed design therefore introduces redundant WAN connectivity and dynamic routing.

The assignment also requires an error-detection or correction scheme, an IPv4 addressing and subnetting plan, a routing-protocol trace, TCP/UDP decisions for each traffic type, congestion-control and QoS mechanisms, an application-layer message exchange, and simulation measurements. These elements are integrated into one coherent architecture.

## 1.1 Restated Problem

The task is to design and evaluate a resilient banking network in which branches, ATMs and the data center can communicate securely and efficiently. The network must continue to carry critical financial traffic even when a primary branch WAN connection fails. The design must also prevent lower-priority traffic such as video from overwhelming transaction traffic.

## 1.2 Design Philosophy

The design follows a layered approach. Physical redundancy is supported by alternate links, the data-link layer detects corrupted frames, the network layer provides subnetting and dynamic routing, the transport layer provides reliable or low-overhead delivery according to application needs, QoS protects critical traffic, and application protocols support services such as DNS and HTTPS.

# 2. Objectives

Design a reliable physical topology for four branches, the ATM network and the central data center.

Provide a primary and backup WAN path for branch connectivity.

Select suitable transmission media and cabling standards.

Demonstrate CRC encoding and detection of a corrupted frame.

Create an IPv4 addressing and subnetting scheme for multiple network segments.

Use OSPF to dynamically discover routes and recover from a WAN failure.

Choose TCP or UDP according to the requirements of transaction, camera and online-banking traffic.

Implement a congestion-control and QoS strategy that gives critical transactions priority.

Identify application-layer protocols and trace an online-banking session.

Simulate normal and degraded network conditions and measure throughput, delay, jitter and packet loss.

Analyze the results, limitations and design decisions, and connect the work to relevant learning outcomes and SDGs.

# 3. Requirements and Environment

| Requirement | Proposed Choice |
| --- | --- |
| Branches | 4 branch offices |
| Central site | Central data center |
| ATM infrastructure | Citywide ATM network |
| Topology | Hybrid star + redundant WAN |
| Simulation | Cisco Packet Tracer |
| Routing | OSPF (Link State) |
| IP version | IPv4 |
| LAN medium | Cat6 UTP |
| Backbone medium | Fiber optic |
| Error detection | CRC |
| Transactions | TCP |
| Online banking | TCP + HTTPS |
| CCTV | UDP |
| QoS | Priority queuing + traffic shaping |
| Required metrics | Throughput, delay, jitter, packet loss |

## 3.1 Environment Used

Cisco Packet Tracer is proposed as the simulation environment because it supports routers, switches, end devices, server services, IP addressing, routing configuration and packet-level observation. The topology can be implemented as a logical representation of the bank's production architecture.

## 3.2 Traffic Parameters

| Traffic Type | Approx. Simulation Priority | Suggested Characteristics |
| --- | --- | --- |
| Bank transaction | Highest | Small packets, reliable TCP, low delay |
| Online banking | High | HTTPS/TCP, interactive |
| CCTV | Medium | Continuous UDP stream, delay-sensitive |
| Email/general data | Low | Best-effort background traffic |
| Backup/file transfer | Lowest | Bulk traffic, can tolerate delay |

# 4. Proposed Network Architecture

The selected architecture is a hybrid topology. Each branch uses a star LAN in which a central switch connects employee computers, ATMs, cameras and other devices. Branch routers connect the LAN to the WAN. Each branch has a primary WAN path and a backup WAN path to maintain service during a primary-link failure.

The central data center contains an edge router, firewall, core switching/routing infrastructure and a server farm. The server farm hosts the banking application, database, web service, DNS, mail and monitoring functions.

The ATM network is treated as a logically separated service segment. In an actual deployment, ATMs would be isolated from ordinary employee devices through VLANs, access-control policies and dedicated security zones.

## 4.1 High-Level Network Diagram

## 4.2 Branch-Level Architecture

Branch Router
                         /          \
                 Primary WAN      Backup WAN
                     |                |
                Data Center      Data Center
                     |
                 Firewall
                     |
                Core Switch
               /    |     \
             PCs   ATMs    CCTV

# 5. Physical Topology and Devices

## 5.1 Why a Hybrid Topology?

A pure bus topology is unsuitable for a banking environment because a shared backbone creates a large dependency on one medium and can make troubleshooting difficult. A simple star topology is effective inside a branch but does not by itself provide WAN redundancy. A full mesh provides excellent path diversity but becomes expensive as the number of sites grows. The hybrid approach combines the practical advantages of star LANs with redundant WAN paths.

## 5.2 Devices at the Branch Tier

| Device | Role | Reason |
| --- | --- | --- |
| Router | WAN/LAN routing | Connects branch subnet to remote networks |
| Managed switch | LAN aggregation | Connects PCs, ATMs and cameras |
| Firewall/security gateway | Traffic filtering | Controls permitted traffic |
| Access point | Controlled wireless access | Supports approved staff devices |
| IP camera | Surveillance | Generates video traffic |

## 5.3 Devices at the Data-Center Tier

| Device | Role |
| --- | --- |
| Edge router | External/WAN connectivity |
| Firewall | Security boundary |
| Core switch/router | High-speed internal forwarding |
| Web server | Online-banking portal |
| Application server | Business logic |
| Database server | Account/transaction data |
| DNS server | Name resolution |
| Monitoring server | Operational visibility |

## 5.4 Device Failure Considerations

Production deployment should avoid single points of failure. Critical routers, firewalls, switches and power supplies can be duplicated. The simulation focuses primarily on the required WAN-failure case, but the architecture is designed so that additional redundancy can be introduced later.

# 6. Transmission Media and Cabling

## 6.1 Cat6 UTP

Cat6 unshielded twisted-pair cable is proposed for branch Ethernet connections to desktop computers, cameras, access points and other local devices. It is practical, relatively inexpensive and easy to terminate and troubleshoot.

## 6.2 Fiber Optic Backbone

Fiber optic cable is proposed for high-speed data-center backbone links and high-capacity interconnection. Fiber provides high bandwidth and immunity to electromagnetic interference, making it suitable for a critical environment.

## 6.3 WAN Connectivity

For simulation, point-to-point Ethernet or serial-style WAN connections can represent the primary and backup services. In a real deployment, dedicated leased lines, Metro Ethernet, MPLS, SD-WAN or other carrier services could be used depending on business and service-provider requirements.

# 7. OSI Layer Mapping

| OSI Layer | Design Application |
| --- | --- |
| 7 – Application | DNS, HTTPS, SMTP, DHCP/NTP services |
| 6 – Presentation | Data formatting and encryption through TLS-related processing |
| 5 – Session | Session establishment and management |
| 4 – Transport | TCP reliability/congestion control; UDP low overhead |
| 3 – Network | IPv4 addressing and OSPF routing |
| 2 – Data Link | Ethernet framing and CRC error detection |
| 1 – Physical | Cat6, fiber and WAN transmission media |

The layered model helps isolate problems. For example, a cable fault is primarily a physical-layer issue, an incorrect subnet mask is a network-layer configuration issue, and excessive TCP retransmission may indicate transport-layer or lower-layer loss/congestion.

# 8. Data-Link Error Detection Using CRC

Cyclic Redundancy Check (CRC) is selected as the error-detection example. The sender treats the data as a binary polynomial, appends zeros, divides by a generator polynomial using modulo-2 arithmetic, and appends the resulting remainder. The receiver performs the same division. A zero remainder indicates that the received bit pattern is consistent with the CRC; a non-zero remainder indicates an error.

## 8.1 Worked Encoding Example

Data bits: 1101011011

Generator: 10011

The generator has five bits, so four zeros are appended to the data.

Dividend: 11010110110000

Modulo-2 division is performed using XOR rather than ordinary subtraction. For this worked example, the remainder is 1110.

Transmitted codeword = 1101011011 + 1110 = 11010110111110

## 8.2 Detection of a Corrupted Frame

Assume one or more bits are changed during transmission. For example, the received sequence is represented as 11010100111110 rather than the original 11010110111110. The receiver divides the received sequence by generator 10011. Because the corruption changes the polynomial, the resulting remainder is non-zero.

Receiver decision: remainder = 0000 means accept the frame as CRC-consistent; remainder ≠ 0000 means reject the frame as corrupted. CRC therefore provides an efficient data-link error-detection mechanism.

## 8.3 Importance for Banking Traffic

CRC does not itself repair a corrupted frame. It identifies the error so that the corrupted frame is not silently accepted. Reliable higher-layer protocols such as TCP can then cause missing information to be retransmitted when appropriate.

# 9. IPv4 Addressing and Subnetting

The proposed private address space is 10.10.0.0/16. It is divided into separate subnets for branches, ATMs, the data center and WAN point-to-point links. This creates clear routing boundaries and makes address management easier.

| Segment | Network | Prefix | Gateway/Use |
| --- | --- | --- | --- |
| Branch 1 LAN | 10.10.1.0 | /24 | 10.10.1.1 |
| Branch 2 LAN | 10.10.2.0 | /24 | 10.10.2.1 |
| Branch 3 LAN | 10.10.3.0 | /24 | 10.10.3.1 |
| Branch 4 LAN | 10.10.4.0 | /24 | 10.10.4.1 |
| ATM Network | 10.10.20.0 | /24 | 10.10.20.1 |
| Data Center | 10.10.30.0 | /24 | 10.10.30.1 |
| Backup WAN B1 | 10.10.100.0 | /30 | Point-to-point |
| Primary WAN B1 | 10.10.101.0 | /30 | Point-to-point |
| Primary WAN B2 | 10.10.102.0 | /30 | Point-to-point |
| Primary WAN B3 | 10.10.103.0 | /30 | Point-to-point |
| Primary WAN B4 | 10.10.104.0 | /30 | Point-to-point |

## 9.1 Example Branch 1 Addressing

| Device | Address |
| --- | --- |
| Branch router LAN interface | 10.10.1.1 |
| Switch management | 10.10.1.2 |
| PC1 | 10.10.1.10 |
| PC2 | 10.10.1.11 |
| Camera | 10.10.1.20 |
| ATM | 10.10.1.30 |

## 9.2 Data Center Addressing

| Server | Address |
| --- | --- |
| Banking application | 10.10.30.10 |
| Database | 10.10.30.20 |
| Web server | 10.10.30.30 |
| DNS server | 10.10.30.40 |
| Mail server | 10.10.30.50 |
| Monitoring server | 10.10.30.60 |

## 9.3 Benefits of Subnetting

Reduces unnecessary broadcast scope.

Separates functional traffic into logical networks.

Makes routing and access control easier.

Provides room for future expansion.

Supports clearer troubleshooting and documentation.

# 10. Routing Design Using OSPF

OSPF is selected as the Link-State routing protocol. Each participating router forms a view of the network topology and calculates routes based on link costs. The design uses a lower OSPF cost on the primary WAN path and a higher cost on the backup path, causing normal traffic to prefer the primary route.

## 10.1 Normal Path

Branch 1 → Primary WAN → Data Center

## 10.2 Backup Path

Branch 1 → Backup WAN → Data Center

## 10.3 Example Routing Table Before Convergence

| Destination | Next Hop | Interface | State |
| --- | --- | --- | --- |
| 10.10.1.0/24 | Direct | LAN | Connected |
| 10.10.101.0/30 | Direct | Primary WAN | Connected |
| 10.10.30.0/24 | — | — | Learning |
| 10.10.20.0/24 | — | — | Learning |

## 10.4 Example Routing Table After Convergence

| Destination | Next Hop | Cost | Path |
| --- | --- | --- | --- |
| 10.10.1.0/24 | Direct | 0 | Branch LAN |
| 10.10.2.0/24 | B1 WAN router | 20 | Primary |
| 10.10.3.0/24 | B1 WAN router | 20 | Primary |
| 10.10.4.0/24 | B1 WAN router | 20 | Primary |
| 10.10.20.0/24 | Data-center router | 30 | ATM segment |
| 10.10.30.0/24 | Data-center router | 20 | Server segment |

## 10.5 Failure and Reconvergence

When the Branch 1 primary WAN interface goes down, OSPF removes the unavailable path from active forwarding calculations after failure detection and route processing. The backup path then becomes the available route. The exact convergence time depends on timer and interface behavior and should be measured in the simulator rather than assumed.

Normal: Branch 1 → Primary WAN → Data Center

Failure: Branch 1 → Backup WAN → Data Center

# 11. Transport-Layer Design

| Traffic | Protocol | Reason |
| --- | --- | --- |
| Bank transaction traffic | TCP | Reliable, ordered delivery, retransmission and congestion control |
| ATM surveillance video | UDP | Low overhead and low delay; occasional packet loss is preferable to retransmission delay |
| Online-banking portal | TCP | Reliable web communication and secure HTTPS transport |

## 11.1 Banking Transactions

A financial transaction must be delivered reliably and in the correct order. TCP provides acknowledgements, sequencing, retransmission, flow control and congestion control. This makes TCP appropriate for transaction messages.

## 11.2 ATM Camera Feed

Camera video is continuous traffic. If an individual packet is lost, retransmitting it may arrive too late to be useful for a real-time stream. UDP avoids TCP's connection and retransmission overhead and can therefore support a low-latency stream.

## 11.3 Online Banking

The online-banking portal uses TCP underneath HTTPS. Reliable delivery is required for login requests, account information and transaction-related application data.

# 12. Congestion Control and QoS

Congestion occurs when offered traffic approaches or exceeds link capacity. Because financial transactions are more important than background traffic, the design applies QoS classification and queueing. Transaction traffic receives the highest priority, online banking receives high priority, CCTV receives controlled medium priority, and background traffic receives lower priority.

## 12.1 Priority Model

| Priority | Traffic | Policy |
| --- | --- | --- |
| 1 – Highest | ATM/banking transactions | Strict or reserved priority |
| 2 – High | Online banking | High-priority queue |
| 3 – Medium | CCTV video | Controlled bandwidth |
| 4 – Low | Email/general traffic | Best effort |
| 5 – Lowest | Bulk backup/file transfer | Use remaining capacity |

## 12.2 Example 100-Mbps Allocation

| Traffic Class | Illustrative Allocation |
| --- | --- |
| Transactions | 30 Mbps |
| Online banking | 30 Mbps |
| CCTV | 25 Mbps |
| Other traffic | 15 Mbps |

These figures are simulation parameters, not measured production requirements. They can be adjusted after observing traffic behavior.

## 12.3 TCP Congestion Control

TCP uses mechanisms such as slow start, congestion avoidance, fast retransmit and fast recovery. When loss suggests congestion, TCP reduces its sending rate. QoS complements TCP by controlling which applications receive bandwidth and queueing preference.

## 12.4 Traffic Shaping

Traffic shaping controls the rate at which traffic enters a link or queue. It prevents a burst of low-priority traffic from consuming all capacity and helps preserve responsiveness for critical transactions.

# 13. Application-Layer Services

| Protocol/Service | Purpose in Proposed Network |
| --- | --- |
| DNS | Resolves the online-banking domain to an IP address |
| HTTPS/HTTP | Web portal communication; HTTPS is used for secure banking |
| SMTP | Email communication |
| DHCP | Automatic address assignment where appropriate |
| NTP | Time synchronization |
| SFTP/FTP | Controlled file transfer where required |
| Application/API services | Business and transaction processing |

For an actual banking environment, unencrypted HTTP should not be used for sensitive authentication or financial information. HTTPS should be the normal customer-facing protocol.

# 14. End-to-End Online Banking Message Trace

## 14.1 Scenario

A customer opens a browser, enters the bank's online-banking domain, logs in and requests an account balance.

## 14.2 DNS

The client first needs the IP address of the banking portal. It sends a DNS query to its configured DNS resolver. The DNS service returns the address of the portal. In the simulation, the web server may use 10.10.30.30.

Client → DNS Server: DNS query for online.meridianbank.example

DNS Server → Client: DNS response containing the portal IP address

## 14.3 TCP Three-Way Handshake

The client establishes a TCP connection:

Client → Server: SYN

Server → Client: SYN-ACK

Client → Server: ACK

## 14.4 TLS and HTTPS

The browser and server establish the cryptographic protection required for HTTPS. After the secure session is established, login information is carried through the encrypted application connection.

## 14.5 Authentication and Balance Request

- Client sends encrypted HTTPS login request.

- Web server forwards the request to the banking application.

- Application server validates credentials and accesses required account information.

- Application server sends a balance request to the database.

- Database returns the requested account information.

- Application server creates the response.

- Web server returns the HTTPS response to the client.

- Browser displays the account balance.

## 14.6 Complete Trace

Customer Browser → DNS → Web Server → Application Server → Database → Application Server → Web Server → Customer Browser

At every stage, routing and security policies determine whether the traffic is permitted. TCP provides reliable delivery, while TLS protects the confidentiality and integrity of the application session.

# 15. Security Design

## 15.1 Firewall

The firewall is placed at the data-center security boundary. It should allow only required services and connections. Unsolicited inbound traffic should be blocked by default, with explicit rules for approved services.

## 15.2 VLAN Segmentation

| VLAN | Purpose | Example |
| --- | --- | --- |
| 10 | Employees | Branch user devices |
| 20 | ATM | ATM systems |
| 30 | CCTV | IP cameras/video |
| 40 | Management | Network administration |

## 15.3 Encryption

Customer-facing banking services should use HTTPS/TLS. Sensitive management traffic should also use secure administrative protocols. Encryption reduces the risk that credentials or financial information can be read by an unauthorized party.

## 15.4 Access Control

Administrative access should be limited to authorized personnel. Device management accounts should use strong authentication and role-based permissions. Network devices should not expose management interfaces unnecessarily.

## 15.5 Monitoring

A monitoring/logging system should collect information about link status, routing changes, authentication events, device health and unusual traffic patterns. This supports troubleshooting and security investigation.

# 16. Simulation Plan

The simulation should be constructed in Cisco Packet Tracer with the topology, addresses, routing and traffic classes described in this report. The team should save the project file and capture screenshots of the completed topology, interface addressing, OSPF operation, routing tables and failure test.

## 16.1 Implementation Steps

- Place routers, switches, servers and end devices.

- Connect devices using appropriate simulated media.

- Assign IPv4 addresses and default gateways.

- Create the branch, ATM and data-center LANs.

- Configure primary and backup WAN links.

- Enable OSPF on the participating routers.

- Verify connectivity with ping and path-tracing tools.

- Configure application servers and DNS.

- Generate or model transaction, web and CCTV traffic.

- Record normal performance.

- Shut down the primary WAN interface of one branch.

- Observe OSPF reconvergence and traffic rerouting.

- Record degraded-link performance.

- Compare the two data sets.

## 16.2 Evidence to Capture

Complete topology screenshot

IP configuration screenshots

OSPF configuration and neighbor information

Routing table before convergence

Routing table after convergence

Successful end-to-end connectivity test

Primary WAN failure screenshot

Backup route in operation

Performance measurements for both scenarios

# 17. Test Cases

| Test Case | Condition | Expected Result | Evidence |
| --- | --- | --- | --- |
| TC1 | Normal network | All major services work | Ping/application output |
| TC2 | Primary B1 WAN down | Backup path selected | Routing table/path |
| TC3 | High transaction traffic | Transactions remain responsive | Delay/loss |
| TC4 | High CCTV traffic | CCTV is controlled by QoS | Queue/traffic results |
| TC5 | CRC corruption | Non-zero remainder detected | Worked calculation |
| TC6 | DNS query | Portal IP returned | DNS output |
| TC7 | Online login | HTTPS service reachable | Browser/server output |
| TC8 | Remote branch access | OSPF route exists | Ping/path output |

# 18. Performance Metrics and Analysis

## 18.1 Throughput

Throughput is the amount of successfully delivered data per unit time. A common expression is Throughput = Successfully received data / elapsed time. Higher throughput generally indicates better utilization, provided that critical traffic remains responsive.

## 18.2 End-to-End Delay

End-to-end delay is the time from source transmission to destination reception. It can include processing, queuing, transmission and propagation components.

## 18.3 Jitter

Jitter is the variation in packet delay. It is particularly relevant to continuous media such as surveillance video.

## 18.4 Packet Loss

Packet loss percentage can be calculated as (lost packets / sent packets) × 100.

## 18.5 Illustrative Expected Results

| Metric | Normal Load | Primary WAN Failure | Interpretation |
| --- | --- | --- | --- |
| Throughput | 92 Mbps | 78 Mbps | Backup route may have lower effective capacity |
| Average delay | 18 ms | 35 ms | Alternate path adds delay |
| Jitter | 3 ms | 8 ms | Rerouting/congestion can increase variation |
| Packet loss | 0.2% | 1.2% | Temporary convergence and congestion can increase loss |

Important: these numbers are illustrative expected values for explaining the analysis. They are not actual simulator measurements. The final submission should replace them with the team's measured results.

## 18.6 Result Comparison

Under normal operation, the primary WAN route is expected to provide the preferred path, resulting in lower delay and higher throughput. When the primary path is removed, OSPF should redirect traffic through the backup link. The network may experience a short disruption during failure detection and reconvergence, followed by continued connectivity.

| Parameter | Normal | Degraded | Expected Observation |
| --- | --- | --- | --- |
| WAN path | Primary | Backup | Route changes |
| Throughput | Higher | Lower | Backup capacity/path |
| Delay | Lower | Higher | Longer route |
| Jitter | Lower | Higher | More queue/path variation |
| Packet loss | Low | Slightly higher | Transient convergence |
| Service availability | High | Maintained | Redundancy works |

# 19. Failure and Recovery Analysis

## 19.1 Primary WAN Failure

The key degraded-link test is the loss of a branch's primary WAN connection. The branch router should detect the interface/link failure, OSPF should update its topology information, and the backup route should become the forwarding path.

## 19.2 Expected Sequence

- Primary WAN link operates normally.

- Primary interface is disabled in the simulator.

- Neighbor relationship or interface state changes.

- OSPF recalculates the affected route.

- Backup path becomes active.

- Traffic resumes through the backup link.

- Performance metrics are collected.

## 19.3 Business Impact

The backup route reduces the chance that a single WAN fault will completely isolate a branch. There may still be a temporary impact during failure detection and reconvergence, so the design should be tested under realistic failure and recovery conditions.

# 20. Advantages and Limitations

## 20.1 Advantages

Redundant WAN connectivity improves availability.

Star branch LANs are simple to troubleshoot.

OSPF dynamically selects routes.

Subnetting provides logical separation.

CRC detects corrupted frames.

TCP provides reliable delivery for financial traffic.

UDP supports low-latency surveillance streaming.

QoS protects critical transaction traffic.

Firewall and VLAN segmentation improve security.

The design can be expanded with additional branches and services.

## 20.2 Limitations

Redundancy increases infrastructure and operating cost.

OSPF convergence is not instantaneous.

Backup links may have lower capacity.

QoS requires careful classification and tuning.

Packet Tracer is a simulation and cannot reproduce every production condition.

Actual WAN performance depends on the service provider and physical infrastructure.

Illustrative performance values must be replaced by measured simulator results.

# 21. SDG Relevance

The assignment identifies SDG 8, SDG 9 and SDG 16 as relevant to the banking-network problem.

## 21.1 SDG 8 – Decent Work and Economic Growth

Reliable financial connectivity supports business activity, employee productivity and access to banking services. A resilient network helps branches and customers continue normal financial operations.

## 21.2 SDG 9 – Industry, Innovation and Infrastructure

The proposed redundant architecture is an example of resilient digital infrastructure. Network automation, dynamic routing, monitoring and traffic management support dependable digital services.

## 21.3 SDG 16 – Peace, Justice and Strong Institutions

Secure financial systems depend on reliable technology and appropriate access control. Protecting banking services contributes to trust in financial institutions.

# 22. Learning Outcomes and Reflection

| Course Outcome | How the Project Demonstrates It |
| --- | --- |
| CO1 | OSI layers, physical media, Ethernet framing and CRC are applied. |
| CO2 | IPv4 subnetting, addressing and OSPF routing are designed. |
| CO3 | TCP, UDP and application protocols are selected and analyzed. |
| CO4 | Hybrid topology, switches, routers, firewalls and cabling are designed. |
| CO5 | Throughput, delay, jitter and packet loss are measured in simulation. |

## 22.1 Reflection

The most important design lesson is that network reliability is not achieved by one feature alone. A redundant link without dynamic routing would require manual intervention, while dynamic routing without adequate capacity and QoS could still leave critical traffic exposed to congestion. Similarly, a reliable transport protocol cannot compensate for a completely unavailable physical path. The final design therefore combines multiple layers.

Another important lesson is the difference between reliability and low latency. Banking transactions need correctness and reliable delivery, whereas live camera feeds can tolerate occasional loss if that avoids excessive delay. This is why the design uses TCP and UDP differently instead of applying one protocol to every application.

The simulation stage is also important because design assumptions need to be tested. Actual measured throughput, delay, jitter, packet loss and convergence behavior should be used to validate or revise the design.

# 23. Individual Contribution

| Group Member | Contribution |
| --- | --- |
| Member 1 | Network topology and physical design |
| Member 2 | IPv4 addressing and subnetting |
| Member 3 | OSPF routing and WAN failure testing |
| Member 4 | TCP/UDP, congestion control and QoS |
| Member 5 | Simulation, performance analysis and report preparation |

Replace the placeholder member descriptions with the actual names and contributions of the group members before submission.

# 24. Conclusion

The proposed Meridian Bank network combines a practical branch star topology with redundant WAN connectivity to create a resilient banking communication system. The central data center provides the main banking, web, database and infrastructure services, while the ATM network and branch LANs are separated into logical addressing domains.

Private IPv4 addressing and subnetting make the network easier to manage and route. OSPF provides dynamic path selection and allows the backup WAN connection to be used when the primary link becomes unavailable. CRC provides a clear data-link method for detecting corrupted frames.

TCP is appropriate for banking transactions and online banking because reliability, sequencing and retransmission are essential. UDP is appropriate for surveillance video because low delay is more important than guaranteed retransmission of every packet. QoS and traffic shaping ensure that critical financial traffic remains responsive when the network is congested.

The online-banking scenario demonstrates how DNS, TCP, TLS/HTTPS, the web server, application server and database interact. The simulation plan then validates the design through normal and degraded-link tests using throughput, delay, jitter and packet loss.

The resulting architecture is not only technically suitable for the assignment but also demonstrates the main principles of reliable network engineering: redundancy, segmentation, dynamic routing, appropriate transport protocols, traffic prioritization, security and measurement.

# 25. References

- 1. Department of Computer Science and Engineering, CSA07 – Computer Networks Assignment Questions, Meridian Bank network-design problem statement and assessment rubric.

- 2. Cisco Networking Academy, networking fundamentals and Cisco Packet Tracer learning resources.

- 3. IEEE 802.3 Ethernet standards and Ethernet networking concepts.

- 4. Internet Engineering Task Force (IETF), RFC 2328 – OSPF Version 2.

- 5. Internet Engineering Task Force (IETF), RFC 791 – Internet Protocol.

- 6. Internet Engineering Task Force (IETF), RFC 793 – Transmission Control Protocol.

- 7. Internet Engineering Task Force (IETF), RFC 768 – User Datagram Protocol.

- 8. Internet Engineering Task Force (IETF), RFC 1034/RFC 1035 – Domain Name System.

- 9. Computer Networks course materials covering OSI model, subnetting, routing, TCP/UDP, congestion control and application protocols.

# Appendix A: Configuration Templates

## A.1 Example IPv4 Router Interface Plan

Branch 1 LAN interface: 10.10.1.1/24

Branch 2 LAN interface: 10.10.2.1/24

Branch 3 LAN interface: 10.10.3.1/24

Branch 4 LAN interface: 10.10.4.1/24

ATM gateway: 10.10.20.1/24

Data-center gateway: 10.10.30.1/24

## A.2 OSPF Conceptual Configuration

The exact interface names depend on the Packet Tracer router model. The conceptual steps are:

- Enter router configuration mode.

- Start an OSPF process.

- Assign a router ID where appropriate.

- Advertise the branch LAN and WAN subnets.

- Set the primary WAN to a lower OSPF cost than the backup WAN.

- Verify neighbors and learned routes.

- Test the route before and after disabling the primary link.

## A.3 Example Verification Commands

show ip interface brief

show ip route

show ip ospf neighbor

show ip ospf interface

ping <destination>

traceroute <destination>

## A.4 Screenshot Checklist

Topology

IP addressing

OSPF neighbors

Routing table before convergence

Routing table after convergence

Normal traffic

Primary WAN failure

Backup route

Performance results

# Appendix B: One-Page Executive Summary

Meridian Bank requires a reliable network connecting four branches, a citywide ATM network, a central data center, surveillance systems and online banking. The proposed solution uses a hybrid topology: star LANs inside branches and redundant WAN paths between branches and the data center.

The private IPv4 address space 10.10.0.0/16 is subnetted into branch, ATM, data-center and point-to-point WAN networks. OSPF is used for dynamic routing. The primary WAN is assigned a lower routing cost, while the backup link is retained as an alternate path. When the primary connection fails, OSPF reconverges and forwards traffic through the backup path.

CRC is used to detect corrupted data-link frames. TCP is selected for banking transactions and online banking, while UDP is selected for surveillance video. QoS gives transaction traffic the highest priority, followed by online banking, CCTV and background traffic. Traffic shaping prevents background applications from consuming the entire WAN capacity.

The online-banking exchange begins with DNS resolution, followed by a TCP handshake and TLS/HTTPS session. The request passes through the web and application layers and accesses the database before the balance is returned to the customer.

The simulation should compare normal operation with a primary-WAN failure using throughput, end-to-end delay, jitter and packet loss. The expected conclusion is that redundancy preserves connectivity while the backup route may introduce additional delay, jitter and reduced throughput. Actual simulator measurements must be inserted into the final results table.