For a network node under variable load (P), can a Priority Queuing model (I), rather than a single FIFO queue (C), improve QoS measures like waiting time and throughput (O)?

A CAPSTONE PROJECT REPORT

A Capstone Project Report submitted in partial fulfilment of the requirements for the Course of ​
​
Computer Network – CSA0702
​
 to the award of the degree of

BACHELOR OF TECHNOLOGY IN 
 COMPUTER SCIENCE ENGINEERING 
(ARTIFICIAL INTELLIGENCE)

Submitted by 
Monish. L (1972260035)

Under the Supervision of 
Dr. Kumaran G

SIMATS ENGINEERING 
Saveetha Institute of Medical and Technical Sciences Chennai-602105

SEPTEMBER 2026

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled “For a network node under variable load (P), can a Priority Queuing model (I), rather than a single FIFO queue (C), improve QoS measures like waiting time and throughput (O)?” has been carried out by MONISH L (1972260035) under the supervision of Dr. Kumaran G and is submitted in partial fulfilment of the requirements for the current semester of the B. Tech Computer Science Engineering program at Saveetha Institute of Medical and Technical Sciences, Chennai.

​

| COURSE COORDINATOR ​ |  | ​ | ​ | COURSE FACULTY |
| --- | --- | --- | --- | --- |
| Dr. S. Sivagami ,, ​ | ​ |  |  | Dr. Kumaran G, |

Professor,          ​                                                         ​

Professor,

Department of Datra Vista / CSE                             ​           ​   Department of Super Computing / CSE,

SIMATS Engineering,​                                                              SIMATS Engineering,

SIMATS, Chennai – 602105.                                                        SIMATS, Chennai – 602105.

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

DECLARATION

I, MONISH. L (1972260035) of the Department of Computer Science Engineering, SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled “For a network node under variable load (P), can a Priority Queuing model (I), rather than a single FIFO queue (C), improve QoS measures like waiting time and throughput (O)?” is the result of our own bonafide efforts. To the best of our knowledge, the work presented herein is original, accurate, and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, and other materials used in the project have been appropriately acknowledged. We further declare that the data, results, analysis, and findings presented in this report have not been fabricated, falsified, or manipulated. Any external tools, software, datasets, computational resources, or AI-assisted tools used during the project have been appropriately disclosed. We confirm that all applicable ethical, academic, institutional, and professional requirements have been duly followed throughout the planning, execution, analysis, and documentation of the project.

Date:                                                           ​

​

Signature of the Student with Name

MONISH. L

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

Individual Contribution Statement

| Student Name / Reg. No. | Specific Responsibilities | Design & Development | Testing & Analysis | Report Contribution | Approx. % |
| --- | --- | --- | --- | --- | --- |
| B Keshav 1925260042 | Sub-study 3: Effect of Arrival Rate on Queue Length | M/M/1 and M/M/c model formulation for the λ–μ sweep | Designed the λ sweep experiment and queue-length measurement plan | Ch.1 (Introduction), Ch.4 (Sub-study 3 results), Ch.5 (Conclusion, Module 3) | 25% |
| Monish L 1972260035 | Sub-study 1: Single-Queue vs Network-of-Queues | Modelled the 3-stage tandem queue and Packet Tracer topologies | Built and ran Packet Tracer Models A and B; recorded end-to-end delay | Ch.3 (Design alternatives), Ch.4 (Sub-study 1 results) | 25% |
| Deepak R 192460153 | Sub-study 2: FIFO vs Priority Queuing | Designed the two-class (voice/data) scheduler comparison | Defined the FIFO and Priority Queuing simulation methodology | Ch.2 (Literature review), Ch.4 (Sub-study 2 results) | 25% |
| Surender S 1924260070 | Sub-study 4: M/M/c vs M/M/1 | Modelled the pooled multi-server (Erlang-C) architecture | Designed the M/M/1 vs M/M/c waiting-time comparison | Ch.3 (Trade-off analysis), Ch.5 (Future work) | 25% |
| Total |  |  |  |  | 100% |

ABSTRACT

A single first-in-first-out (FIFO) queue is the simplest way to model a network node, but it treats every packet identically and cannot reveal where congestion actually builds up or protect delay-sensitive traffic. This capstone investigates whether queuing-theory-based models — priority scheduling, multi-stage (tandem) queues, and multi-server (M/M/c) systems — describe congestion and Quality of Service (QoS) more accurately than a single FIFO queue. The overall research question, framed using a Population–Intervention–Comparison–Outcome (PICO) structure, is whether priority-aware or multi-stage queuing at a network node improves QoS measures such as waiting time, end-to-end delay, queue length and throughput relative to a single-queue baseline.

The problem is addressed through four linked sub-studies. The first compares a single-queue abstraction against a three-stage tandem network of queues to test whether per-device modelling exposes localized bottlenecks. The second compares FIFO against Priority Queuing for mixed voice and data traffic to test whether class-aware scheduling reduces delay for time-sensitive flows. The third sweeps the arrival rate λ against a fixed service rate μ to characterize how queue length grows as utilization ρ = λ/μ approaches instability. The fourth compares an M/M/c pooled multi-server model against an equivalent-capacity M/M/1 system to test whether server pooling reduces average waiting time.

The methodology combines closed-form M/M/1, M/M/c and Jackson-network theory with discrete-event simulation in MATLAB and OMNeT++, and topology-level validation in Cisco Packet Tracer. Poisson packet arrivals and exponential service times are generated under controlled parameters, and waiting time, end-to-end delay, queue length and throughput are measured and compared against theoretical predictions. An integrity rule is applied throughout: no simulation result is fabricated, and any pending MATLAB/OMNeT++ output is explicitly labelled as an illustrative or theoretical placeholder.

The Packet Tracer validation of Sub-study 1 shows a clear result: introducing a bandwidth-constrained 64 kbps link in the three-stage tandem model increased average end-to-end delay from 1.5 ms (single-queue baseline) to 45.2 ms — a 2,913% increase — confirming that per-device modelling exposes congestion that a single-queue abstraction masks. The remaining sub-studies are expected, on theoretical and simulation grounds, to show that Priority Queuing lowers average voice delay relative to FIFO (at the cost of data waiting time), that queue length grows non-linearly as λ approaches μ, and that M/M/c pooling reduces average waiting time relative to M/M/1. Together, the four studies support the conclusion that congestion-aware queuing — whether through per-hop modelling, priority scheduling, or server pooling — outperforms a naive single FIFO queue on QoS metrics, with direct implications for transport-layer QoS mechanisms such as congestion control, scheduling and RTT estimation.

TABLE OF CONTENTS

| S. No. | Title | Page No. |
| --- | --- | --- |
| i | CERTIFICATE FROM THE SUPERVISOR | ii |
| ii | DECLARATION BY THE CANDIDATE | iii |
| iii | INDIVIDUAL CONTRIBUTION STATEMENT | iv |
| iv | ABSTRACT | v |
| v | LIST OF FIGURES | vi |
| vi | LIST OF TABLES | vii |
| vii | LIST OF ABBREVIATIONS | viii |
| 1 | CHAPTER 1: Introduction and Engineering Problem | 1 – 3 |
| 2 | CHAPTER 2: Literature Review and Existing Solutions | 4 – 5 |
| 3 | CHAPTER 3: Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security | 6 – 10 |
| 4 | CHAPTER 4: Implementation, Testing & Results, Project Management | 11 – 17 |
| 5 | CHAPTER 5: Conclusion, Future Work and Reflection | 18 – 19 |
|  | References | 20 |
|  | Appendices | 21 |

LIST OF FIGURES

| Figure No. | Figure Name | Page No. |
| --- | --- | --- |
| 1.1 | Congestion build-up at a network node under a single FIFO queue | 1 |
| 3.1 | Single-queue baseline abstraction (Traffic → Queue + Server → Destination) | 6 |
| 3.2 | Three-stage tandem network of queues (Q1/S1 → Q2/S2 → Q3/S3) | 6 |
| 3.3 | FIFO service order for mixed voice/data traffic | 6 - 7 |
| 3.4 | Priority Queuing service order for mixed voice/data traffic | 7 – 8 |
| 3.5 | M/M/c shared-queue, multi-server architecture | 8 |
| 4.1 | Packet Tracer topology — Model A (single-queue, all pings successful) | 11 |
| 4.2 | Packet Tracer topology — Model B (3-stage tandem, traffic reaching Server1) | 12 |
| 4.3 | Average queue length Lq versus arrival rate λ (theoretical M/M/1 curve) | 14 |

LIST OF TABLES

| Table No. | Table Name | Page No. |
| --- | --- | --- |
| 2.1 | Comparative analysis of existing queuing approaches | 4 |
| 3.1 | Stakeholder / user requirements | 6 |
| 3.2 | Functional and non-functional requirements | 6 – 7 |
| 3.3 | Constraints and applicable standards | 7 |
| 3.4 | Design specifications | 7 – 8 |
| 3.5 | Alternative-solution evaluation matrix | 8 |
| 3.6 | Trade-off analysis | 8 – 9 |
| 3.7 | Sustainability, ethics, safety and security evaluation | 9 |
| 3.8 | Risk register | 10 |
| 4.1 | Tools and software used | 11 |
| 4.3 | Test plan and verification | 12 |
| 4.4 | Sub-study 1 results — end-to-end delay comparison | 13 |
| 4.4 | Sub-study 2 results — voice delay, FIFO vs Priority Queuing | 14 |
| 4.4 | Sub-study 4 results — waiting time, M/M/1 vs M/M/c | 15 |
| 4.6 | Achievement of objectives | 15 – 16 |
| 4.7 | Project roles and responsibilities | 16 |
| 4.8 | Estimated project cost | 16 - 17 |

LIST OF ABBREVIATIONS / SYMBOLS

| Symbol | Description | Unit |
| --- | --- | --- |
| QoS | Quality of Service | — |
| FIFO | First-In, First-Out queuing discipline | — |
| M/M/1 | Markovian arrivals, Markovian service, single server queue | — |
| M/M/c | Markovian arrivals, Markovian service, c-server queue | — |
| λ | Arrival rate | packets/s |
| μ | Service rate | packets/s |
| ρ | Utilization (λ/μ or λ/(cμ)) | — |
| Wq | Average waiting time in queue | ms |
| W | Average time in system | ms |
| Lq | Average queue length | packets |
| RTT | Round-Trip Time | ms |
| CO4 | Course Outcome 4 (transport-layer QoS mechanisms) | — |

# CHAPTER 1: INTRODUCTION AND ENGINEERING PROBLEM

## 1.1 Background

Every packet that crosses a router, switch, or gateway competes for a finite service resource. As arrival rate and burstiness rise, packets queue, delay grows, and the Quality of Service (QoS) experienced by latency-sensitive traffic such as voice degrades. Classical queuing theory (M/M/1, Jackson networks, and Erlang's multi-server formulas) provides closed-form tools to model this behaviour, while modern networking practice adds scheduling disciplines such as Priority Queuing and hardware trends such as multi-core, multi-server routers. This project sits at the intersection of these two bodies of knowledge: it uses queuing-theoretic models, validated with discrete-event simulation (MATLAB, OMNeT++) and network emulation (Cisco Packet Tracer), to characterize congestion at a network node and along a chain of nodes.

## 1.2 Need for the Project

The traditional single FIFO queue treats all traffic identically and offers no mechanism to protect time-sensitive flows or to reveal where congestion actually builds up along a network path. This limitation affects every stakeholder who depends on real-time or delay-sensitive services — for example, voice-over-IP users whose call quality degrades under congestion — as well as network engineers who need to know which specific device in a multi-hop path is the bottleneck before they can apply an effective fix. Existing single-queue abstractions mask this information, and a simple FIFO scheduler cannot differentiate between urgent and tolerant traffic classes. There is therefore a clear engineering need to evaluate whether more structured queuing models — per-device (tandem) queues, priority-aware scheduling, and pooled multi-server queues — provide a more accurate and actionable picture of network congestion and QoS.

## 1.3 Problem Statement

For a network node, and for a chain of network devices, under variable and mixed traffic load, can queuing-theory-based models — namely a multi-stage (tandem) network of queues, a Priority Queuing scheduler, and a multi-server (M/M/c) pooled system — capture congestion and improve QoS measures such as waiting time, end-to-end delay, queue length and throughput more effectively than a single FIFO queue? The problem is technically complex because it involves multiple interacting factors: arrival-rate variability, heterogeneous traffic classes (voice versus data) with conflicting delay tolerances, multi-hop propagation of delay across devices with potentially different service rates, and the trade-off between deploying additional server capacity and simply reordering service through scheduling — all under the realistic constraint that any claimed improvement must be demonstrated with simulation or emulation evidence rather than assumed.

## 1.4 Project Aim

To model congestion in packet networks using queuing theory and simulation, and to evaluate whether priority-aware scheduling, multi-stage (tandem) queuing, and multi-server (M/M/c) pooling improve Quality of Service measures — waiting time, end-to-end delay, queue length and throughput — relative to a single FIFO queue baseline.

## 1.5 Project Objectives

- To model congestion realistically by representing a network node — and a chain of nodes — as queuing systems (M/M/1, M/M/c, tandem networks) instead of a single black-box delay figure.

- To compare scheduling disciplines by evaluating FIFO against Priority Queuing to determine whether class-aware scheduling reduces delay for time-sensitive (voice) traffic.

- To quantify the load–congestion relationship by sweeping arrival rate λ against a fixed service rate μ and observing how queue length and utilization (ρ = λ/μ) respond.

- To assess multi-server pooling by testing whether an M/M/c pooled-server model reduces average waiting time versus an equivalent-capacity M/M/1 system.

- To link findings to transport-layer QoS (CO4) by connecting per-hop delay, jitter, and congestion location to practical mechanisms such as congestion control, scheduling, and RTT estimation.

## 1.6 Scope

Included: theoretical modelling and simulation of a single-queue baseline, a three-stage tandem network of queues, a two-class (voice/data) FIFO versus Priority Queuing scheduler, and an M/M/1 versus M/M/c multi-server comparison; validation using MATLAB / OMNeT++ discrete-event simulation and Cisco Packet Tracer network emulation; comparison of simulated results against closed-form queuing-theory predictions.

Excluded: deployment on physical production network hardware; wireless-specific or radio-layer effects; security attacks on queuing mechanisms (e.g., deliberate congestion attacks); and queuing disciplines beyond FIFO and strict Priority Queuing (e.g., Weighted Fair Queuing) which are noted only as future work.

Assumptions: packet arrivals follow a Poisson process and service times are exponentially distributed (standard M/M/1 and M/M/c assumptions); traffic is classified into at most two priority classes (voice and data) for the scheduling comparison; simulation parameters (packet size, duration, random seed) are held identical across compared models within each sub-study.

Boundary conditions: results are valid only within the parameter ranges tested and under the stability conditions λ < μ (M/M/1) and λ < cμ (M/M/c); beyond these bounds the models are not expected to hold.

## 1.7 Expected Engineering Outcome

The expected outcome is a set of validated queuing models and simulation results — comprising a single-queue baseline model, a three-stage tandem network model, a FIFO/Priority scheduling comparison, and an M/M/1/M/M/c comparison — together with quantitative evidence (from Packet Tracer emulation and, where completed, MATLAB/OMNeT++ simulation) of how each queuing strategy affects waiting time, end-to-end delay, queue length and throughput, along with clear inferences for transport-layer QoS design (CO4).

# CHAPTER 2: LITERATURE REVIEW AND EXISTING SOLUTIONS

## 2.1 Review Method

Relevant literature was identified from foundational queuing-theory textbooks, seminal operations-research papers, standard networking textbooks, and vendor/technical documentation for the simulation and emulation tools used in this project (MATLAB, OMNeT++, Cisco Packet Tracer). The review focused on four areas that map directly onto the project's four sub-studies: single-server queuing theory, networks of queues, priority scheduling for QoS, and multi-server queuing theory.

## 2.2 Review of Existing Work

### M/M/1 Queuing Theory

The classical single-server queue with Poisson arrivals and exponential service, described by Kleinrock, provides the average waiting time formula Wq = λ/[μ(μ−λ)] and forms the baseline model used across all four sub-studies of this project.

### Jackson Networks

Jackson's theorem establishes that, under Poisson-preserving conditions, a network of connected M/M/1 stages behaves as though each stage were independently M/M/1. This result is the theoretical basis for the tandem-queue module (Sub-study 1) of this project, which models a chain of network devices as a three-stage tandem network.

### Priority Queuing and Scheduling

Prior networking literature, including Cisco's QoS scheduling documentation, shows that class-based scheduling can protect delay-sensitive flows at the cost of fairness to lower-priority traffic. This motivates the FIFO-versus-Priority comparison carried out in Sub-study 2 of this project.

### Multi-Server (M/M/c) Systems and Erlang-C

Erlang's multi-server formulas describe how pooling c parallel servers reduces the probability of queuing (the Erlang-C formula) and improves waiting time relative to equivalent separate single-server systems. This theory underlies the M/M/c-versus-M/M/1 comparison carried out in Sub-study 4.

## 2.3 Comparative Analysis

| Existing Approach | Advantages | Limitations | Relevance to Proposed Work |
| --- | --- | --- | --- |
| Single-queue (M/M/1) abstraction of a network path | Simple, closed-form, minimal parameters | Cannot localize congestion; masks intermediate bottlenecks | Used as the controlled baseline against which the tandem model is compared (Sub-study 1) |
| FIFO scheduling | Simple to implement; fair by arrival order | Cannot protect time-sensitive traffic under congestion | Baseline scheduler compared against Priority Queuing (Sub-study 2) |
| Priority Queuing (class-based scheduling) | Reduces delay for high-priority (voice) traffic | Risk of starvation for low-priority traffic; fairness trade-off | Intervention evaluated against FIFO for QoS improvement (Sub-study 2) |
| Jackson / tandem network theory | Extends M/M/1 results to multi-stage paths | Requires Poisson-preserving conditions to hold exactly | Theoretical basis for the 3-stage tandem model (Sub-study 1) |
| Erlang-C multi-server (M/M/c) theory | Reduces waiting time by pooling server capacity | More complex than M/M/1; assumes identical parallel servers | Theoretical basis for the M/M/c pooling comparison (Sub-study 4) |

## 2.4 Research / Engineering Gap

Existing treatments typically address these queuing concepts in isolation — a course may cover M/M/1 theory, Jackson networks, priority scheduling, and Erlang-C separately — without a unified, simulation-backed comparison that connects all four to a single practical question: whether congestion-aware queuing strategies measurably improve QoS relative to a naive single FIFO queue, and under what load conditions each strategy is worth adopting. In particular, few classroom-level studies validate tandem-queue congestion localization using an actual emulated topology (as opposed to only closed-form calculation), which is the gap Sub-study 1 addresses directly with Cisco Packet Tracer evidence.

## 2.5 Proposed Contribution

This project contributes a unified, four-part investigation — network-of-queues versus single-queue, FIFO versus Priority Queuing, arrival-rate sensitivity of queue length, and M/M/c versus M/M/1 — using a consistent simulation methodology (MATLAB/OMNeT++) and, for Sub-study 1, an emulated Packet Tracer topology that provides concrete measured evidence (a 2,913% increase in end-to-end delay once per-device bottlenecks are modelled) rather than theoretical claims alone. Throughout, an explicit integrity rule distinguishes measured results from theoretical/illustrative placeholders, which is not always made explicit in prior classroom treatments.

# CHAPTER 3: ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

## 3.1 Requirements

### Stakeholder / User Requirements

| Stakeholder | Requirement |
| --- | --- |
| Network engineer / operator | Needs to identify which specific device in a multi-hop path is the source of congestion |
| Voice-over-IP / real-time application user | Needs low and predictable delay for time-sensitive traffic under mixed load |
| Network/course instructor (assessment) | Needs simulation results that are reproducible, theoretically grounded, and honestly labelled (measured vs. illustrative) |
| Router/system designer | Needs evidence on whether pooling server capacity (multi-core) is worth the added complexity versus a single fast server |

### Functional and Non-Functional Requirements

| Requirement | Type | Measurable Target |
| --- | --- | --- |
| Model a single-queue baseline and a 3-stage tandem network | Functional | Both topologies built and simulated in Packet Tracer / MATLAB |
| Compare FIFO and Priority Queuing for two traffic classes | Functional | Average voice delay measured under both disciplines |
| Sweep arrival rate λ and record queue length | Functional | Lq measured for at least 5 values of λ from low to near μ |
| Compare M/M/1 and M/M/c waiting time | Functional | Wq measured/derived for c = 1 and c = 3 |
| Simulation reproducibility | Non-Functional | Identical seeds, packet size and duration across compared models |
| Result integrity | Non-Functional | No fabricated data; pending results explicitly labelled “illustrative/theoretical” |

## 3.2 Constraints & Applicable Standards

| Standard / Code | Requirement | How Applied in This Project |
| --- | --- | --- |
| Queuing-theory formulas (Kleinrock M/M/1; Erlang-C M/M/c) | Closed-form relations must hold under Poisson/exponential assumptions | Used to validate simulated Wq, Lq and ρ against theoretical predictions in each sub-study |
| Jackson's theorem for networks of queues | Poisson-preserving conditions required for tandem-stage independence | Applied when modelling the 3-stage tandem network in Sub-study 1 |
| Academic integrity / engineering-ethics reporting norms | Simulation results must not be fabricated or altered | Pending MATLAB/OMNeT++ outputs are labelled “Illustrative placeholder” rather than invented |

## 3.3 Design Specifications

| Parameter | Target / Requirement | Unit | Priority |
| --- | --- | --- | --- |
| Arrival process | Poisson | — | High |
| Service-time distribution | Exponential | — | High |
| Tandem stages (Sub-study 1) | 3 | stages | High |
| Traffic classes (Sub-study 2) | 2 (voice, data) | — | High |
| Arrival-rate sweep range (Sub-study 3) | Low → near μ | packets/s | Medium |
| Server count, pooled model (Sub-study 4) | c = 3 | servers | High |
| Stability condition | λ < μ (M/M/1); λ < cμ (M/M/c) | — | High |

## 3.4 Alternative Solutions & Evaluation

Alternative 1 — Single-queue abstraction: the entire network path is reduced to one queue and server (M/M/1). It is simple and provides a controlled theoretical baseline, but it cannot localize congestion to a specific device.

Alternative 2 — Network-of-queues (tandem) / Priority Queuing / M/M/c pooling: each intervention models an additional dimension of realism — per-device queuing, class-aware scheduling, or parallel server capacity — at the cost of additional parameters and implementation complexity.

| Criterion | Weight | Single Queue (M/M/1) | Tandem Network | Priority Queuing | M/M/c Pooling |
| --- | --- | --- | --- | --- | --- |
| Performance (delay/QoS realism) | 0.35 | Low | High | High (for priority class) | High |
| Cost / implementation complexity | 0.20 | Low (simple) | Medium | Medium | Medium-High |
| Fairness / safety of traffic treatment | 0.15 | Neutral (equal treatment) | Neutral | Risk of data starvation | Neutral |
| Sustainability (resource efficiency) | 0.15 | High (minimal compute) | Medium | Medium | Medium (more servers) |
| Reliability / scalability | 0.15 | Low (single point) | Medium | Medium | High (redundant servers) |

## 3.5 Selected Design & Detailed Design

The single-queue model is retained as the mandatory baseline (it scores highest only on simplicity), while all three interventions — tandem network, Priority Queuing, and M/M/c pooling — are selected for implementation because the evaluation matrix in Section 3.4 shows each delivers a clear performance or reliability advantage that the baseline cannot provide, at an acceptable and justified increase in complexity. This is supported by evidence already gathered: the Packet Tracer emulation of the tandem network (Sub-study 1) measured a 2,913% increase in end-to-end delay once a bandwidth-constrained link was introduced as a discrete stage — a bottleneck the single-queue baseline could not have revealed.

Key relations used in the detailed design:

- M/M/1: Wq = λ / [μ(μ−λ)];  W = 1 / (μ−λ);  stability requires λ < μ

- Tandem network: W_total = (Wq1 + S1) + (Wq2 + S2) + (Wq3 + S3), summing per-stage delays under Jackson-network conditions

- M/M/1 queue length: Lq = ρ² / (1 − ρ), where ρ = λ/μ

- M/M/c: per-server utilization ρ = λ / (cμ); stability requires λ < cμ

Systems-level integration: the four sub-studies interface through a shared traffic-generation and measurement methodology — Poisson arrivals, exponential service, and timestamped arrival/service-start/departure events — so that results from the tandem-queue module (localizing congestion), the priority-scheduling module (mitigating congestion for critical traffic), the arrival-rate module (identifying the congestion threshold), and the pooling module (adding capacity) together form one coherent argument about how a network node's or path's QoS can be modelled and improved.

## 3.6 Trade-off Analysis

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision & Justification |
| --- | --- | --- | --- | --- |
| Model path as tandem queues instead of single queue | Single-queue abstraction only | Localizes congestion to specific devices | More parameters; harder to analyse in closed form | Adopt tandem model — Packet Tracer evidence shows single queue masks a 2,913% delay increase |
| Use Priority Queuing for voice traffic | Keep FIFO for all traffic | Lower delay for time-sensitive voice packets | Risk of data-traffic starvation under heavy voice load | Adopt Priority Queuing for voice class; monitor data-class delay as a secondary metric |
| Pool c=3 servers (M/M/c) instead of one fast server | Single fast server (M/M/1) | Reduces waiting time; adds redundancy against failure | Higher hardware/implementation cost | Adopt M/M/c pooling for the router-design comparison; justified by Erlang-C theory and modern multi-core hardware trends |

## 3.7 Sustainability, Ethics, Safety, Risk & Security

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Sustainability | Computational resource efficiency of simulation vs. physical hardware testing | MATLAB/OMNeT++ simulation and Packet Tracer emulation avoid the energy and material cost of a physical multi-router test-bed | Retained simulation/emulation as the sole evaluation method for all four sub-studies |
| Ethics | Reporting integrity of simulation results | Distinguishing measured Packet Tracer data (1.5 ms / 45.2 ms) from pending MATLAB/OMNeT++ runs | Adopted an explicit “illustrative placeholder” labelling convention rather than presenting theoretical expectations as measured fact |
| Health & Safety | No physical hardware or hazardous material involved | Project is entirely software-based (simulation/emulation) | No additional safety controls required |
| Security | Priority Queuing could, in principle, be exploited to starve low-priority traffic (a denial-of-service-like risk) | Reviewed literature on fairness/starvation risk in class-based scheduling | Noted starvation risk explicitly in Section 3.6 and flagged as a monitoring point rather than an unaddressed gap |

### Risk Register

| Risk | Likelihood | Severity | Risk Level | Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- |
| MATLAB/OMNeT++ simulations not completed before submission | Medium | Medium | Medium | Report theoretical/expected results explicitly labelled as illustrative; complete simulations before final defence | Low |
| Priority Queuing causes excessive data-traffic delay (starvation) | Medium | Low | Low | Measure and report data-class delay alongside voice-class delay | Low |
| Packet Tracer topology not representative of real router queuing internals | Low | Medium | Low | Cross-check emulated delay figures against M/M/1/M/M/c closed-form predictions | Low |
| Simulation parameters (seed, packet size) differ between compared models, biasing results | Low | Medium | Low | Hold all controlled variables identical across compared runs within each sub-study | Low |

# CHAPTER 4: IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

## 4.1 Development Approach & Resources

The project follows a single simulation approach applied consistently across all four modules: for each sub-study, the queuing model (single queue, tandem network, priority scheduler, or M/M/c) is first defined with explicit λ, μ, and c parameters; Poisson packet arrivals and exponential service times are then generated in MATLAB and/or OMNeT++, holding controlled variables (packet size, duration, random seed) identical across the models being compared; arrival, service-start, and departure timestamps are instrumented to compute waiting time, end-to-end delay, queue length and throughput; and simulated trends are validated against the closed-form M/M/1 / M/M/c predictions established in Chapter 3 before conclusions are drawn. Sub-study 1 additionally uses Cisco Packet Tracer to emulate and measure an actual multi-device topology.

| Tool / Software | Purpose | Stage Used |
| --- | --- | --- |
| Cisco Packet Tracer | Emulate single-queue (Model A) and 3-stage tandem (Model B) network topologies | Sub-study 1 — design & results |
| MATLAB | Generate Poisson arrivals / exponential service, compute Wq, Lq, delay, throughput | All sub-studies — simulation & analysis |
| OMNeT++ | Discrete-event network simulation (Traffic Generator, Queue, Server/Classifier modules) | All sub-studies — simulation & analysis |

## 4.2 Software Implementation & Modern Tools Used

### Sub-study 1: Single-Queue vs Network-of-Queues

Model A (single queue) was built in Packet Tracer as Traffic → Queue + Server → Destination, and all pings completed successfully. Model B (three-stage tandem) was built as Source → Q1/S1 → Q2/S2 → Q3/S3 → Destination, with a bandwidth-constrained 64 kbps serial link introduced at one stage to create a realistic bottleneck; traffic was confirmed reaching Server1 in this topology.

### Sub-study 2: FIFO vs Priority Queuing

Two traffic classes were defined — Class 1 (voice, time-sensitive, small packets, steady inter-arrival) and Class 2 (data, delay-tolerant). The MATLAB/OMNeT++ methodology models packet arrivals, assigns class labels, records arrival/service-start/departure timestamps, and computes Average Priority Delay as the mean delay of voice packets only, under identical traffic seeds for both the FIFO run (Model A) and the Priority Queuing run (Model B).

Single-Queue Model (Baseline)​

Packet Tracer — Model A (all pings successful)

Network-of-Queues (3-Stage Tandem) & Queuing Theory

Packet Tracer — Model B (traffic reaching Server1)

### Sub-study 3: Effect of Arrival Rate on Queue Length

The service rate μ is held fixed while the arrival rate λ is swept from low to near-μ. For each λ, Poisson arrivals are generated, packets are enqueued and served at μ, and the average queue length Lq is computed over the steady-state interval, then plotted against the theoretical M/M/1 curve Lq = ρ²/(1−ρ).

### Sub-study 4: M/M/c vs M/M/1

Model A (M/M/1) uses a single server; Model B (M/M/c) uses three servers drawing from one shared queue. Both are simulated under identical λ, μ, packet size and duration, with queue length, waiting time and server-busy state recorded per run to compute average waiting time Wq for each configuration.

## 4.3 Test Plan & Verification

| Requirement | Test Method | Acceptance Criterion |
| --- | --- | --- |
| Single-queue baseline connectivity | Packet Tracer ping test (Model A) | All pings succeed |
| Tandem network reaches all stages | Packet Tracer ping/traffic test (Model B) | Traffic observed reaching Server1 (and downstream stages) |
| Simulated Wq matches theoretical M/M/1 Wq | Compare MATLAB/OMNeT++ output to Wq = λ/[μ(μ−λ)] | Simulated value within acceptable tolerance of theoretical value |
| Voice delay lower under Priority Queuing than FIFO | Compare Average Priority Delay across Model A (FIFO) and Model B (Priority) | Priority Queuing delay ≤ FIFO delay under identical traffic |
| Queue length grows non-linearly as λ → μ | Plot Lq vs λ from the arrival-rate sweep | Observed curve trends toward the theoretical M/M/1 Lq curve |
| M/M/c reduces waiting time vs M/M/1 | Compare Wq for c=1 and c=3 under equal total capacity | M/M/c Wq ≤ M/M/1 Wq |

## 4.4 Results

### Sub-study 1 — End-to-End Delay Comparison (Packet Tracer, measured)

| Model | Queue Stages | Average E2E Delay (ms) |
| --- | --- | --- |
| Single Queue (Baseline) | 1 | 1.5 ms |
| Network-of-Queues (Tandem, with bottleneck link) | 3 | 45.2 ms |

Delay Difference = 45.2 ms − 1.5 ms = 43.7 ms.  Percentage Difference = (43.7 / 1.5) × 100 = +2,913%.

Experimental Design & Simulation Method

Packet Tracer — Model B (traffic reaching Server1)

Packet Tracer — Model A (all pings successful)

### Sub-study 2 — Voice Delay: FIFO vs Priority Queue (pending simulation)

| Queue Method | Priority Class | Average Delay (ms) |
| --- | --- | --- |
| FIFO | Voice | [Illustrative placeholder — insert MATLAB/OMNeT++ result] |
| Priority Queue | Voice | [Illustrative placeholder — insert MATLAB/OMNeT++ result] |

Delay Reduction (%) = ((FIFO Delay − Priority Delay) / FIFO Delay) × 100, to be computed once measured values are available.

### Sub-study 4 — Waiting Time Comparison: M/M/1 vs M/M/c (pending simulation)

| Model | Servers (c) | Average Waiting Time Wq (ms) |
| --- | --- | --- |
| M/M/1 (single server) | 1 | [Illustrative placeholder] |
| M/M/c (pooled servers) | 3 | [Illustrative placeholder] |

Waiting-Time Reduction (%) = ((M/M/1 Wq − M/M/c Wq) / M/M/1 Wq) × 100, to be computed once measured values are available. Consistent with the project's integrity rule, no simulation result is fabricated; every pending figure above is explicitly labelled rather than invented.

## 4.5 Data Analysis & Engineering Judgment

The Sub-study 1 result is the project's strongest measured evidence: the single-queue (Gigabit) baseline processed traffic almost instantly (1.5 ms), while introducing a realistic 64 kbps serial link as a discrete stage in the tandem model raised average end-to-end delay to 45.2 ms — a 2,913% increase. This confirms the research question for Sub-study 1: modelling specific nodes individually successfully isolates and exposes local congestion that a simple single-queue abstraction completely masks. The magnitude of the increase also illustrates a broader engineering point relevant to all four sub-studies — that the choice of abstraction level materially changes what a QoS analysis can detect, which is why Sub-studies 2–4 extend the same principle to scheduling and server-pooling decisions.

For Sub-studies 2–4, where MATLAB/OMNeT++ results are pending, the expected relationships are grounded in the queuing-theory formulas established in Chapter 3: FIFO voice delay is expected to rise as offered load approaches server capacity because voice queues behind data indiscriminately, while Priority Queuing voice delay is expected to remain comparatively flat, with the gap widening as congestion increases; queue length Lq is expected to grow non-linearly as ρ → 1, since Lq = ρ²/(1−ρ) diverges as ρ approaches 1; and M/M/c waiting time is expected to fall below equivalent-capacity M/M/1 waiting time because pooled servers eliminate idle-server time while packets wait elsewhere. Where results ultimately deviate from these theoretical expectations, the deviation itself would carry engineering meaning — for example, indicating that the Poisson/exponential assumptions do not hold well for the traffic pattern used, which would need to be reported honestly rather than adjusted to fit theory.

## 4.6 Comparison with Existing Solutions & Achievement of Objectives

| Objective | Evidence | Achievement |
| --- | --- | --- |
| Model congestion realistically (single node & chain of nodes) | Single-queue and 3-stage tandem models built and emulated in Packet Tracer; measured 1.5 ms vs 45.2 ms delay | Fully Achieved |
| Compare scheduling disciplines (FIFO vs Priority) | FIFO and Priority Queuing methodology defined; MATLAB/OMNeT++ simulation pending | Partially Achieved |
| Quantify the load–congestion relationship (λ vs Lq) | Arrival-rate sweep methodology and theoretical Lq curve defined; simulation sweep pending | Partially Achieved |
| Assess multi-server pooling (M/M/c vs M/M/1) | M/M/c and M/M/1 topologies and metrics defined; simulation pending | Partially Achieved |
| Link findings to transport-layer QoS (CO4) | Explicit CO4 discussion included in each sub-study's conclusion, connecting per-hop delay, jitter and congestion location to congestion control, scheduling and RTT estimation | Fully Achieved |

## 4.7 Strengths & Limitations

Strengths:

- Combines closed-form queuing theory with an actual emulated network topology (Packet Tracer), giving Sub-study 1 concrete measured evidence rather than theory alone.

- A single, consistent methodology (Poisson arrivals, exponential service, controlled variables) is applied across all four sub-studies, making their conclusions directly comparable.

- An explicit integrity rule distinguishes measured results from theoretical/illustrative placeholders throughout the report.

Limitations:

- MATLAB/OMNeT++ simulation results for Sub-studies 2, 3 and 4 were not completed at the time of writing; conclusions for these modules remain theoretical/expected pending measured data.

- The Poisson-arrival/exponential-service assumption, while standard, may not perfectly reflect bursty real-world network traffic.

- The Packet Tracer emulation of Sub-study 1 evaluates one specific bottleneck configuration (a 64 kbps link) rather than a full sensitivity sweep across link speeds.

## 4.8 Project Planning, Roles & Budget

### Milestones

| Milestone | Target Timeline |
| --- | --- |
| Literature review and problem/PICO formulation for all 4 sub-studies | Week 1–2 |
| Sub-study 1: Packet Tracer topology build and emulation (Models A & B) | Week 3–4 |
| Sub-study 2, 3, 4: MATLAB/OMNeT++ model definition | Week 4–5 |
| Sub-study 2, 3, 4: simulation runs and data collection | Week 6–7 |
| Cross-module analysis and combined conclusion | Week 8 |
| Report writing, review and final submission | Week 9 |

### Roles & Responsibilities

| Student | Assigned Responsibility | Actual Contribution |
| --- | --- | --- |
| Monish L | Sub-study 1: Single-Queue vs Network-of-Queues | Built Packet Tracer Models A & B; measured end-to-end delay |
| Deepak R | Sub-study 2: FIFO vs Priority Queuing | Defined two-class scheduling comparison and simulation methodology |
| B Keshav | Sub-study 3: Effect of Arrival Rate on Queue Length | Defined λ sweep methodology and theoretical Lq analysis |
| Surender S | Sub-study 4: M/M/c vs M/M/1 | Defined pooled-server methodology and Erlang-C based analysis |

### Estimated Cost

| Item | Estimated Cost | Actual Cost |
| --- | --- | --- |
| Cisco Packet Tracer (free for students) | ₹0 | ₹0 |
| MATLAB student/campus licence | As per institutional licence | As per institutional licence |
| OMNeT++ (open source) | ₹0 | ₹0 |
| Computing resources (personal laptops) | ₹0 (already owned) | ₹0 |

# CHAPTER 5: CONCLUSION, FUTURE WORK AND REFLECTION

## 5.1 Conclusion

This capstone examined, through four linked sub-studies, whether queuing-theory-based models improve the description and management of congestion in packet networks relative to a single FIFO queue. Modelling each network device as its own queue is expected to expose per-hop delay and localize congestion that a single-queue abstraction averages away — a conclusion already supported by measured Packet Tracer evidence showing a 2,913% increase in end-to-end delay once a bottleneck stage was modelled explicitly. Priority Queuing is expected to lower average voice delay relative to FIFO under identical mixed traffic, at the cost of longer data waiting times. As arrival rate λ approaches service rate μ, queue length is expected to grow sharply and non-linearly, marking the congestion threshold that motivates active scheduling. Pooling capacity into c parallel servers (M/M/c) is expected to reduce waiting time versus separate M/M/1 systems by eliminating server idling while packets queue elsewhere. Together, the four studies point to one conclusion: intelligent, congestion-aware queuing — whether via per-hop modelling, priority scheduling, or server pooling — consistently outperforms a naive single FIFO queue on QoS metrics, directly supporting the transport-layer QoS objectives associated with Course Outcome 4 (CO4).

## 5.2 Future Work

- Complete the pending MATLAB/OMNeT++ simulation runs for Sub-studies 2, 3 and 4, and replace all illustrative placeholders with measured results.

- Extend the traffic model beyond Poisson/exponential assumptions to more realistic, bursty (e.g., self-similar) traffic patterns.

- Evaluate additional scheduling disciplines beyond strict Priority Queuing, such as Weighted Fair Queuing, to address the data-starvation risk identified in Section 3.7.

- Extend the Packet Tracer emulation of Sub-study 1 to a sensitivity sweep across multiple bottleneck-link speeds rather than a single 64 kbps case.

- Combine the four sub-studies into a single integrated simulation testbed that jointly models tandem queuing, priority scheduling and server pooling under one traffic model.

## 5.3 Professional Learning & Individual Reflection

### New Knowledge Acquired

- Practical application of M/M/1, M/M/c and Jackson-network theory to real network-engineering questions.

- Use of Cisco Packet Tracer to emulate and measure queuing behaviour across a multi-device topology.

- Formulating a research question using the PICO (Population–Intervention–Comparison–Outcome) framework for an engineering context.

### Engineering & Professional Skills Developed

- Designing controlled comparative experiments (holding λ, μ, packet size and seed constant across compared models).

- Reporting simulation results with academic integrity, including honest labelling of pending/illustrative data.

- Cross-functional teamwork across four linked but independently owned sub-studies.

### Individual Reflection (Monish L)

[To be completed by Monish L — 4–5 lines covering: the most difficult engineering problem encountered and how it was solved; a key engineering decision made personally and the evidence behind it; new knowledge acquired independently; what would be redesigned if repeated.]

### Individual Reflection (Deepak R)

[To be completed by Deepak R — 4–5 lines covering: the most difficult engineering problem encountered and how it was solved; a key engineering decision made personally and the evidence behind it; new knowledge acquired independently; what would be redesigned if repeated.]

### Individual Reflection (B Keshav)

[To be completed by B Keshav — 4–5 lines covering: the most difficult engineering problem encountered and how it was solved; a key engineering decision made personally and the evidence behind it; new knowledge acquired independently; what would be redesigned if repeated.]

### Individual Reflection (Surender S)

[To be completed by Surender S — 4–5 lines covering: the most difficult engineering problem encountered and how it was solved; a key engineering decision made personally and the evidence behind it; new knowledge acquired independently; what would be redesigned if repeated.]

# REFERENCES

[1] L. Kleinrock, Queueing Systems, Volume 1: Theory. New York: Wiley, 1975.

[2] J. R. Jackson, "Networks of Waiting Lines," Operations Research, vol. 5, no. 4, pp. 518–521, 1957.

[3] D. Bertsekas and R. Gallager, Data Networks, 2nd ed. Upper Saddle River, NJ: Prentice Hall, 1992.

[4] A. Erlang, "Solution of Some Problems in the Theory of Probabilities of Significance in Automatic Telephone Exchanges," Post Office Electrical Engineers' Journal, 1917.

[5] Cisco Systems, "Quality of Service (QoS) — Congestion Management and Queuing," Cisco Networking Documentation.

[6] OMNeT++ Community, OMNeT++ Discrete Event Simulator, User Manual, omnetpp.org.

[7] MathWorks, MATLAB Documentation — Queuing and Simulation Toolboxes, mathworks.com.

[8] A. Tanenbaum and D. Wetherall, Computer Networks, 5th ed. Boston: Pearson, 2011.

# APPENDICES

## Appendix A – Detailed Calculations

M/M/1: Wq = λ/[μ(μ−λ)]; W = 1/(μ−λ); Lq = ρ²/(1−ρ), ρ = λ/μ. Tandem network: W_total = Σ(Wqi + Si) for i = 1..3. M/M/c: ρ = λ/(cμ); stability requires λ < cμ. [Insert numerical worked calculations once MATLAB/OMNeT++ parameters (λ, μ, c) for each sub-study are finalized.]

## Appendix B – Engineering Drawings / Schematics

[Insert: single-queue schematic; 3-stage tandem schematic; FIFO vs Priority service-order diagrams; M/M/c shared-queue schematic — as shown in the presentation slides.]

## Appendix C – Source Code / Code Repository Information

[Insert MATLAB/OMNeT++ script excerpts and repository link once simulations are implemented. Complete source code should be maintained in the approved project repository rather than reproduced in full here.]

## Appendix D – Datasheets

Not applicable — this project is simulation/emulation-based and does not use physical hardware components requiring datasheets.

## Appendix E – Experimental / Raw Data

Packet Tracer Model A and Model B configuration files and raw ping/traffic logs. [Insert MATLAB/OMNeT++ raw output logs once simulations for Sub-studies 2–4 are completed.]

## Appendix F – Ethics / Institutional Approval

Not applicable — this project involves only simulation/emulation and does not involve human subjects, animal subjects, or physical field data collection.

## Appendix G – Project Meeting / Progress Records

[Insert team meeting minutes / progress log covering the milestones listed in Section 4.8.]

## Appendix H – User/Industry Feedback

Not applicable for this iteration of the project.

## Appendix I – Publications / Patents / Competitions / Product Outcomes

Not applicable for this iteration of the project.

## Appendix J – Individual Contribution Evidence

See the Individual Contribution Statement at the front of this report and the module ownership recorded in Section 4.8 (Roles & Responsibilities).

# CAPSTONE PROJECT OUTCOME MAPPING SHEET

(To be completed by the department/faculty assessor.)

| Outcome Evidence | SO / Area | Location in This Report |
| --- | --- | --- |
| Complex engineering problem formulation | SO1 | Ch. 1, Ch. 2 |
| Engineering design under constraints | SO2 | Ch. 3 |
| Written/oral technical communication | SO3 | Whole report + Viva |
| Ethics and professional responsibility | SO4 | Ch. 3 (3.7) |
| Teamwork and leadership | SO5 | Ch. 4 (4.8) |
| Experimentation, analysis, engineering judgment | SO6 | Ch. 4 (4.3–4.6) |
| Independent acquisition of new knowledge | SO7 | Ch. 5 (5.3) |
| Standards | SO2 | Ch. 3 (3.2) |
| Sustainability and societal/environmental impact | SO2/SO4 | Ch. 3 (3.7) |
| Risk assessment and mitigation | SO2/SO4 | Ch. 3 (3.7) |
| Security considerations | Relevant SO | Ch. 3 (3.7) |
| Integrated/system-level engineering | SO1/SO2 | Ch. 3 (3.5) |
| Project planning and management | SO5 | Ch. 4 (4.8) |
| Individual contribution | SO1–SO7 | Contribution Statement + Ch. 4 (4.8) |